CITY OF BAXTER

REQUEST FOR BIDS - CONCRETE REPLACEMENT PROJECT

4/25/2023

INTRODUCTION

The City of Baxter is seeking qualified contractors to bid on a concrete replacement project located at 203 S. Main St. Baxter, Iowa. The bidder will need to summit three bids for the project that must include replacing the following concrete area:

Baxter City Hall:  203 S. Main Street

#1.  This bid must include the following:

- Replace concrete starting at the building to the approach including the sidewalk and side door

- This bid is a patch that is 28ft X 18ft and will include 11ft X 4ft concrete in front of the side door, as well as, a   7ft X 4ft sidewalk replacement patch

- Installation of pipe for the down spout and connect it to the existing down spout pipe (approximately 40ft of 6” PVC)

- No approach should be included in this bid

*The total area is approximately 583 square feet.

#2.  This bid must include the following:

- Replace concrete starting at the building to the approach including the sidewalk and side door

- Installation of pipe for the down spout and connect it to the existing down spout pipe (approximately 40ft of 6” PVC)

- No approach should be included in this bid

*The total area is approximately 1,645 square feet.

#3.  This bid must include the following:

- Replace concrete starting at the building to S. Main St. including the sidewalk and side door

- This bid must include the approach and a flare on the south side of the approach

- Installation of pipe for the down spout and connect it to the existing down spout pipe (approximately 40ft of 6” PVC)

*The total area is approximately 3,055 square feet.

PROJECT DETAILS:  The successful bidder will be responsible for the following:

- Proper sidewalk and driveway barricades to ensure no one enters the worksite

- Demolition, removal, and disposal of the existing concrete

- Preparation of minimum 2” road stone as the base material

- Installation of #4 bar and installed 24” on center

- Installation of 8” 4,000 PSI concrete

- Proper curing of the new concrete

- Final cleanup and restoration of the work area

BID SUBMISSION:  Bids must be submitted in accordance with the instructions in this RFB. The following information must be included:

- Bidder's name, address, phone number, and email address

- Detailed description of the proposed work

- Total cost of the project

- Any additional costs or fees that may be required

- Any exceptions or exclusions to the project requirements

BID DEADLINE:  All bids must be submitted to Baxter City Hall no later than 4:30 pm on Tuesday, May 23, 2023. Late bids will not be considered. Bids can be dropped off in a sealed envelope at 203 S. Main St or e-mailed to maintenance@baxter-iowa.com.

BID EVALUATION:

The City of Baxter will evaluate all bids in accordance with its procurement policies and procedures. The successful bidder will be notified after the bid deadline.

CONTRACT TERMS

The successful bidder will be required to provide the City of Baxter with the following:

- Scope of work

- Project timeline

- Warranty and guarantee information

- Insurance information

CONTACT INFORMATION--All inquiries and requests for additional information should be directed to:

Bryce Halter

641-521-1040

maintenance@baxter-iowa.com

City of Baxter

REQUEST FOR BIDS

CONCRETE REPLACEMENT PROJECT

BID FORM

Submit to Baxter City Hall no later than Tuesday, May 23rd at 4:30 PM.

Contractor Name: __________________________________________________________

Contact Name:	     __________________________________________________________

Contact Phone #:   __________________________________________________________

Address:	      _________________________________________________________

_________________________________________________________

Bid Price for project #1 as outlined in the above introduction:  Bid Price: $ _________________________

Bid Price for project #2 as outlined in the above introduction:  Bid Price: $ _________________________

Bid Price for project #2 as outlined in the above introduction:  Bid Price: $ _________________________

The City of Baxter reserves the right to reject any and all bids.

____________________________________________ Date: __________________________

Authorized Contractor’s Representative Signature